SOLUTIONS PLUS CHEMICAL DATA

BULK GLASS CLEANER

GLASS CLEANER

# DESCRIPTION

BULK GLASS CLEANER is a concentrated mixture of cleaners and surfactants for cleaning glass surfaces.  This product, when diluted to use concentration, leaves a streak free glass surface.

PROPERTIES

Appearance:					Clear Blue Liquid

Odor:							Mild

pH concentrate:					9.3

Solubility:						Complete

Specific Gravity:					0.975

lbs/Gal.:						8.13

USING PROCEDURES

Use diluted at 1:10 to 1:20 parts water up to 1 to 50 parts water depending on the severity of the soil on the glass.

The bulk concentrate in the drum should be hooked up in the plant to obtain the correct dilution.

The product as used should be wiped dry on the glass with absorbent paper.

# Other Areas For Using Bulk Glass Cleaner

Refrigerators, ships, shop floors, shop rags, walls, showers, stoves, air ducts, acoustic tiles, buses, trucks, trains, cars, floor scrubbers, leather, plastics, grills, floors, printing presses, counter tops, venetian blinds, air planes, light fixtures, machinery, tools, steam cleaning.

Directions:  Dilute with water to apply to surface by means of the following:  sprayers, mops, automatic floor scrubbers, sponges, rags or towels.

General Cleaning:  One cup of cleaner to a pail of water, this is approximately 1:20.

Good for walls, trains, buses, floors etc.

Heavy Duty Cleaning: Two cups of cleaner to a pail, this is approximately 1:10.

Good for machinery, tools, light fixtures, printing presses, acoustic tiles, drill bits, air pumps, milling machines, sump tanks etc.

Heavy Carbonized Soils & Baked On Soils, grease, oils:

Add five cups of cleaner to a pail of water.

this is approximately 1:5 solution.

Good for air ducts, grilles baked on carbon, garage floors, range hoods, stoves, wax.

Wax Stripping:

Add four cups of cleaner to a pail of water, agitate surface with brush, scrubbing pad mop or rag, allow to soap and then rinse with warm or hot water for best results.  Do not let solution dry on surface, if solution does dry on surface, repeat above wax stripping procedure.