# SOLUTIONS PLUS CHEMICAL DATA

CONCRETE 60

CONCRETE CLEANER

DESCRIPTION

CON-CRETE 60 is a low pH acid liquid vehicle cleaner used to clean calcium soils from vehicles and to brighten aluminum surfaces.  It is safe on paint, glass, aluminum and Plexiglas surfaces.  CON-CRETE 60 can be used by itself or in conjunction with any of the high pH product lines to produce a two-step “shock treatment.”  CON-CRETE 60 is especially effective for removing cement dust and heavy soils from cement trucks and equipment.

## PROPERTIES

# Appearance:					Clear Water White Liquid

Odor:						Acidic

Foam:						Medium to High

Solubility:					Complete

Flash Point:					None

Specific Gravity:					1.080

lbs/Gal.:					9.01

USING PROCEDURES

For cleaning cement trucks (e.g. concrete mixers), CON-CRETE 60 should be used full strength.  It should be applied by low pressure spray method and should be rinsed off to prevent excessive attack on bare metal surfaces.

To use CON-CRETE 60 as the first part of a two-step “shock treatment,” dilute it from 1 to 10 up to 1 to 20 with water and apply on the vehicle by spray.

Then apply Truck Wash Plus-C at 1:200 to neutralize surface.