SOLUTIONS PLUS CHEMICAL DATA

EXTREME CLEAN PLUS

Vehicle Cleaner/All Purpose Cleaner

DESCRIPTION

EXTREME CLEAN PLUS is a concentrated, heavy duty alkaline cleaner with excellent cleaning and rinsing properties.  Especially formulated to remove a wide variety of soils from processing equipment, floors, and walls found in dairies, meat packing, canning, and beverage plants as well as trucks, locomotives, passenger cars, automobiles.  Removes bugs from automobiles and trucks, that have been on the road and have dried on the surface.  Removes carbon and oils from engine blocks and incrusted grease and is biodegradable.  EXTREME CLEAN PLUS also lubricates brushes on automatic truck, bus and car washes.

PROPERTIES

Form:							Clear liquid

Color:							Pale Yellow

Odor:							Mild

pH (@1%):						10.9

Solubility:						Complete

Phosphates:						1% as elemental phosphorus

Flash Point:						None

Density:						9.01 lbs./gal.

USING PROCEDURES

Heavy soils 1:3 to 1:10, Medium soils 1:10 to 1:20, Light soils 1:25 to 1:75

General cleaning, 1:50 to 1:100.

DISPENSING

Chemical spray arches, steam guns, high pressure spray guns, brush.

Cleaning:  High soil lifting and emulsification properties.  Goes to work quickly and effectively.  Removes a variety of soils including food residues and over-the-road soils (oil, road grime, exhaust film).

Fortified:  Contains special selected detergents, corrosion inhibitors, organic solvents and water softening agents.  Quickly penetrates, loosens soil and suspends dirt so that it can be easily rinsed away.

Rinsing:  Rinses rapidly, freely and completely.

Safe To Use:  Use on aluminum, brass, concrete, cooper, galvanized, glass, iron, plastic, steel, tin, and zinc.

EXTREME CLEAN PLUS can be used on most paints found in the transportation industry and will not adversely affect glass or Plexiglas windows.

DIRECTIONS FOR USE

Hand brushing concrete:  Use at a concentration of 1 once to 6 ounces per gallon of water in a bucket or C.O.P. vat brush.  Rinse with plain water.

Spray Cleaning Exteriors of Filters, Vats, Tanks and Walls:  Rinse thoroughly to remove all loose soil.  Apply the cleaning solution containing 1 to 2 ounces per gallon of hot water through high pressure spray equipment.  Rinse thoroughly with potable water before solution dries.  Sanitize if desired and rinse with potable water if required.

Spray Cleaning Floors:  Remove all solid waste.  Apply the solution containing 1 to 4 ounces per gallon of 140 F to 170 F. water through high pressure equipment.  Extra heavy soil deposits can be brushed with a stiff bristle broom.  After the soil has been loosened, dissolved, or penetrated, rinse with high pressure clean water.

Automatic Floor Scrubbers:  This method is particularly recommended for cleaning heavily soil floors.  Use 2 to 6 ounces per gallon of water.

Trucks, Trailers, Buses, Locomotives, Passenger Cars:  Spray onto units and let soak for one minute, then rinse.  Can be used with Shock System, spray on SP 9300 or SP 9100, let soak for one minute, then spray over top with no rinse in-between with EXTREME CLEAN PLUS, then rinse.

Steam Cleaning:  Use at 1:30 to 1:100 depending on soil.