# SOLUTIONS PLUS CHEMICAL DATA

## NITRO BLAST

VEHICLE CLEANING/GENERAL PURPOSE

Non-Phosphate, Brushless Truck, Trailer, and Locomotive Cleaner

DESCRIPTION

NITRO BLAST is a heavy-duty, liquid alkaline cleaner used for trucks, locomotives, railcars, and passenger cars.  In addition, it provides a wide range of uses in the railway, transportation, and industrial marketplace.  NITRO BLAST was specifically designed to penetrate and remove the toughest greases and road soil, including exhaust.  It is an excellent steam cleaner, locomotive cleaner and can be used for floor and pit cleaning, all surfactants are biodegradable, penetration is rapid, and it is free rinsing.  NITRO BLAST will not keep oils or grease emulsified. NITRO BLAST will not cause problems in the wastewater treatment plant like other cleaners that have high solvent or caustic content.

NITRO BLAST removes road soil even with systems that do not have brushes, including bugs on the windshields and vehicles.

PROPERTIES

# Appearance:					Clear Amber

Odor:						Mild

Foam:						Medium

pH:						13.0

Solubility:					Complete

Flash Point:					None

USING PROCEDURES

Apply to all truck’s surfaces with light pressure.

Spray from bottom to top of truck.  Brush if needed. Dwell time should be 1 to 2 minutes, do not allow dry on surface. Rinse thoroughly.

Light Soils: 64:1

Medium Soils to Heavy Soils: 40-1 or 3-4 oz/gal.

Nitro Blast can also be foamed onto surfaces.

DISPENSING

Chemical spray arches, steam guns, high pressure spray guns, brush or hand spraying.

MATERIAL SAFETY

Safe on iron, steel, stainless steel, rubber, neoprene, nylon and plastic.  Pretest on paint, aluminum, and galvanized.