# SOLUTIONS PLUS CHEMICAL DATA

SUPER LUSTER CLEAN

DESCRIPTION

SUPER LUSTER CLEAN is a liquid, specialized blend of acids and surfactants designed for use in the Trucking, Railway, and Transportation Industry.  It can be used by itself to remove build-up from exhaust fumes, petroleum oils and metallic oxides from painted surfaces, aluminum and steel. SUPER LUSTER CLEAN removes oxide films and brightens aluminum trailers and aluminum railcars.

Note: pre-test on polished aluminum before using.  Not safe on glass or chrome.

## PROPERTIES

# Appearance:					Clear liquid

Odor:							Mild

pH (concentrate)					2.5

Foam:						Medium

Solubility:						Complete

Phosphates:						None

Flash Point:						None

Specific Gravity:					1.068

Emulsification:  					Rapid, complete

USING PROCEDURES

Heavy soils Full Strength to 1:10, Medium soils 1:10 to 1:20, Light soils 1:25 to 1:75.  General cleaning, 1:50 to 1:100.

DISPENSING

Chemical spray arches, spray guns, brush or soak.

_____________________________________________________________

Solutions Plus, Inc., 3907 Bach Buxton Road, Amelia, OH 45102

513-943-9600  Fax  513-943-9600